PROJECT REPORT

MTH-307L Numerical Computation Lab

Solving Fluid Flow in a Pipe Network using Numerical Methods

Submitted By:

Azeem Mohamed Husain

B22F0759CS142

CS Fall22 (Blue)

Submitted To:

Dr. Muhammad Tayyab

Mrs. Iqra Ayesha

Submitted On:

8th of December 2024

Introduction

- Fluid flow analysis in pipe networks is a vital part of engineering, especially when designing systems like water distribution networks and gas pipelines. These networks need to strike a balance between efficiency and functionality while ensuring stability under different operating conditions. Much like electrical grids, the behaviour of fluid flow in pipe networks can be modelled using a set of non-linear algebraic equations. These equations illustrate the relationships between flow rates, pressure losses, and other factors, offering insights into system performance and highlighting potential inefficiencies.

- Numerical methods are essential for solving these non-linear equations, as they often resist analytical solutions. Among these methods, Newton's method is particularly notable for its robustness and quick convergence when applied correctly. This iterative approach is effective for finding the roots of equations, especially in systems with non-linear behaviours, such as fluid flow.

- This project centres on analysing a two-node pipe network where the flow rate is influenced by the pressure difference (dP) through the equation 
Q = K(dP)^ (1/2). The pressure at the supply node remains constant, while the pressure at the receiving node is determined using the derived equations. By applying Newton's method, this study aims to:

- Calculate the pressure at the receiving node.

- Explore how the solution behaves with different initial guesses and flow coefficients (K).

- Extend the analysis to find the maximum K value for which Newton's method still converges.

- This report outlines the formulation of the mathematical model, the derivation of equations, the numerical solution using Newton's method, and the results of the convergence analysis. The findings will offer practical insights into efficiently solving similar non-linear systems.

Model Formulation

- The two-node pipe network under consideration consists of:

- A supply node with a constant pressure (Ps = 100psi)

- A receiving node where the pressure (P) is to be calculated.

- A pipe connecting the nodes, where fluid flow occurs with a flow rate

- The pressure loss (dP) across the pipe is modelled using the following relationship:

dP =Ps -P

- Where dP represents the difference in pressure between the supply and receiving nodes.

- The flow rate (Q) through the pipe depends on AP, and this relationship is given by:

Q = K (dP)^ (1/2)

- Where K is the flow coefficient that accounts for pipe characteristics such as diameter, length, and roughness.

- (dP)^ (1/2) reflects the dependency of flow rate on the square root of the pressure difference.

- To solve for P at the receiving node, we substitute dP =Ps -P into the flow equation, resulting in:

Q = K (Ps - P) ^ (1/2)

- In this project, the pressure at the receiving node (P) is determined by formulating the problem as a non-linear equation:

f(P) = 0

- where f(P) is derived from the relationship between Q and dP

- This mathematical model forms the basis for applying Newton's method to solve for P, given initial conditions and specified tolerance values. The iterative numerical approach ensures the solution satisfies the required accuracy for the pressure at the receiving node.

Derive Equation

- The goal is to derive the non-linear equation f(P)=0 that represents the pressure at the receiving node in the two-node pipe network. Below are the steps:

Step 1: Pressure Loss Relationship

- The pressure loss (dP) across the pipe is defined as:

dP =Ps -P

Step 2: Flow Rate Equation

- The flow rate (Q) through the pipe is related to the pressure difference (dP) by:

​ Q = K (dP)^ (1/2)

- where K is the flow coefficient.

- Substituting dP=Ps−P into the equation gives:

Q = K (Ps - P) ^ (1/2)

Step 3: Non-Linear Equation for PPP

- Rewriting the equation in terms of f(P), the system becomes:

f(P)= Q - K (Ps - P) ^ (1/2) =0

- Here, Q is the flow rate (can be specified or assumed based on the problem requirements), and the equation f(P)=0must be solved numerically to determine PPP.

Step 4: Iterative Solution Using Newton's Method

- To solve f(P)=0 Newton's method requires the derivative f ’(P)):

- f(P)= Q - K (Ps - P) ^ (1/2)

- Differentiate f(P) with respect to P:

f'p=-K2Ps-P

- The iterative formula for Newton's method is then given by:

Pn+1=Pn-fPnf'Pn

- where P(n) is the nth approximation of P, and the iterations continue until the solution converges within a specified tolerance.

Final Non-Linear Equation

- The derived non-linear equation to solve for PPP is:

f(P)= Q - K (Ps - P) ^ (1/2)

- This equation serves as the basis for implementing Newton’s method to find the pressure PPP at the receiving node.

Numerical Solution

- To solve the derived non-linear equation f(P)= Q - K (Ps - P) ^ (1/2) =0 Newton’s method is applied. Below are the detailed steps:

Step 1: Recap of Newton’s Method

- Newton's method is an iterative approach to find roots of non-linear equations. The general formula for updating the approximation P(n)​ is:

Pn+1=Pn-fPnf'Pn

- f(P(n)) The function value at the current guess P(n)

- f ′(P(n)) The derivative of the function at P(n)

- The iterations stop when the absolute difference between successive approximations is less than a specified tolerance (ϵ), or when 
∣ f(P(n)) ∣< ϵ.

Step 2: Define the Problem

- For the system:

f(P)= Q - K (Ps - P) ^ (1/2)

- the derivative is:

f'p=-K2Ps-P

- Substitute these into the Newton’s method formula:

Pn+1=P(n)-Q-KPs-P(n)-K2Ps-P(n)

- Simplify:

Pn+1=Pn+2(Q-KPs-Pn)(Ps-P(n))K

Step 3: Initial Guess and Tolerance

- Initial guess: P0=90 psi (as specified).

- Tolerance: ϵ=10^{-6}

Step 4: Iterative Procedure

- Start P0=90 psi.

- Compute f(P)= Q - K (Ps - P) ^ (1/2) and f'p=-K2Ps-P

- Update P(n)​ using:

Pn+1=Pn+2(Q-KPs-Pn)(Ps-P(n))K

- Check convergence:

If ∣ P(n+1)−P(n) ∣<ϵ, stop the iteration.

If ∣ f(P(n)) ∣<ϵ, stop the iteration.

- Otherwise, set P(n)=P(n+1) and repeat.

Step 5: Analyse Convergence Behaviour

- Try different initial guesses (P0=80,95) to study how the method behaves.

- Observe how the convergence changes with varying K.

Step 6: Extension Task

- Vary the flow coefficient K and find the maximum K where the method converges. For very high values of K, convergence may fail, indicating the method's sensitivity to problem parameters.

Code Implementation

import math # Constants P_s = 100 # Supply node pressure (psi) K = 1.0 # Flow coefficient (adjust as needed) Q = 10 # Flow rate (adjust as needed) tol = 1e-6 # Tolerance max_iter = 100 # Maximum iterations # Newton's method def newton_method(P0): if P0 >= P_s: raise ValueError("Initial guess P0 must be less than P_s to ensure (P_s - P) is positive.") if K <= 0 or Q <= 0: raise ValueError("Flow coefficient (K) and flow rate (Q) must be positive.") P = P0 for i in range(max_iter): f = K * math.sqrt(P_s - P) - Q if P_s - P <= 0: raise ValueError(f"Invalid value encountered: P_s - P = {P_s - P} at iteration {i+1}.") f_prime = -K / (2 * math.sqrt(P_s - P)) P_next = P - f / f_prime # Check convergence if abs(P_next - P) < tol: return P_next, i + 1 P = P_next raise ValueError("Newton's method did not converge within the maximum number of iterations.") # Test the method initial_guess = 90 # Initial guess for P try: P_solution, iterations = newton_method(initial_guess) print(f"Solution for P: {P_solution:.6f} psi (Converged in {iterations} iterations)") except ValueError as e: print(str(e))